chapter

4

Unconventional Gas

Projects

Unconventional resource plays in South Australia are described in Chapter 2. The regulation of unconventional gas projects is covered in detail in Chapter 5. This

chapter describes publicly available details of unconventional gas projects in South Australia, and Figure 4.1 shows the petroleum infrastructure to give a context for these projects. Included are descriptions of the following projects:

•  Santos operated Cooper Basin JV Unconventional Gas Project (Cooper Basin). Deep coal, shale gas and basin- centred gas targets.

•  Beach Energy Nappamerri Trough Project (Cooper Basin). Shale gas and basin-centred gas targets.

•  Senex Energy Unconventional Gas Project (Cooper Basin). Deep coal, shale gas and basin-centred gas targets.

•  Strike Energy Southern Cooper Coal Seam Gas Project (Cooper Basin). Deep and shallow coal seam gas and shale gas targets.

•  Somerton Energy Otway Basin Project.

Shale gas and tight gas targets.

•  Altona Energy and CNOOC Arckaringa

Coal-to-Liquids and Power Project.

•  Sapex Ltd (Linc Energy Ltd) Walloway Basin Underground Coal Gasification Project.

•  Hybrid Energy (Strike Energy) Kingston Coal Project for the production of synthesis gas as feedstock for the manufacture of alternative products including but not limited to: fertiliser, methanol, and synthetic transport fuel.

4.1 Unconventional Gas Projects, Cooper and Otway Basins

4.1.1 Cooper Basin JV Unconventional

Gas Project

Licences And Licence Holders

PPLs 7, 8, 9, 11, 101, 102, 113

Santos Ltd (66.6%)*

Delhi Petroleum Pty Ltd (20.21%) Origin Energy Resources Ltd (13.19%)

*Denotes operator

Location

Cooper Basin, South Australia (Figure 4.2)

Project Summary

Santos, as operator of the Cooper Basin JV, commenced Australia’s first shale gas assessment project in 2006, acquiring a 9.4 m Roseneath Shale core in the Moomba 175 gas development well. Since that time shale cores have been acquired in two other gas development wells (Moomba 185, Bob’s Well

2) for shale gas assessment purposes. Santos has stated that results from the Moomba 185 shale core analyses are commensurate with US producing shale gas basins.

Assessment of the deep coal seam gas potential of the basin commenced with desorption analysis of a 4m Patchawarra coal seam cored in the Dorodillo 2 gas appraisal well in1998.

Further assessment of deep coal seam gas was suspended until 2007, when Santos flowed gas to surface at 100,000 scf/day from a fracture stimulated Patchawarra

Gove

Timor Sea

Papua New Guinea

")

Darwin

Northwest

Shelf

Port Hedland

") ")

Dampier

Broome  ")

") Telfer

NORTHERN   TERRITORY

Alice Springs

Mt Isa

")

QUEENSLAND

") Townsville

CH4

") Newman

# # 	")

Rockhampton

")

Mereenie

Palm Valley

")Gladstone

Geraldton   ")

Beharra Springs

WESTERN AUSTRALIA

") Kalgoorlie

Amadeus

Cooper

Eromanga

Moomba

SOUTH AUSTRALIA

Olympic  Dam ")

ð

Eromanga

#

Ballera

Jackson

R^om#a ")

#

#

Moonie

ð

") Brisbane

ð

Perth  ")

Bunbury  ")

Esperance ")")

Port August#a

Port Bonython ^

NEW SOUTH WALES

ð

")

Newcastle

")

Albany

ADELAIDE ")

ACT  ")

")

ð

Sydney

ð

Katnook

VICTORIA

")

Albury

Canberra

0	500	1000 km

Melbourne

#

Mount Gambier	     Iona	)

Otway	")

Orbost

Gas pipeline, existing Oil–liquids pipeline Ethane pipeline

Equidistant Conic

ð Refinery

^ LPG plant

# Gas and oil treatment plant

Warrnambool

Minerva

(Otway)

Longford

Gippsland

Bass Strait

TASMANIA

Gas pipeline, proposed

Gas treatment plant

") Hobart

204218-0112

Figure 4.1	Map of Australia showing oil and gas infrastructure and Cooper and Otway basins.

Formation coal in the Moomba 77 gas development well. Since then additional deep coal seam gas assessment work has been carried out in seven gas development wells in the South Australian part of the Cooper Basin.

In December 2011, Santos drilled the first dedicated vertical shale gas well (Moomba 191), acquiring core from three shale intervals. A three stage fracture stimulation of the Moomba 191 Roseneath- Epsilon-Murteree (REM) section resulted

in a stabilised flow rate of 2.6 mmscf/d dry gas during the two week flow-back period. Moomba 191 was completed and connected to the Moomba processing facilities in October 2012. This brought the first shale gas production to eastern Australian gas markets.

Santos is accelerating unconventional gas

exploration in 2012-2013, with six vertical

wells planned to test the Nappamerri Trough basin-centred gas play (Figures 4.4 and 4.5), a vertical well to test the deep coal seam gas play at Moomba, and a horizontal well to assess the REM shale play at Moomba North (Figures 4.3 and 4.5).

At end 2010 Santos has booked a total 2C (contingent, probable, recoverable, Santos share) unconventional gas resource of 2,345

PJ, independently certified by DeGolyer and MacNaughton (approximately 3,500 PJ for the Cooper Basin JV). Santos has a 2015 2C target of 4,800 PJ (approximately 7,200 PJ

for the Cooper Basin JV). Santos estimate a potential range for recoverable raw gas from its licences in the Cooper Basin between a

26°0'S

139°30'E

QUEENSLAND

4

26°0'S

141°0'E

Wangkangurru/Yarluyandi Petroleum Conjunctive Indigenous Land Use Agreement (SI2011/023)

SC97/3

The Wangkangurru/Yarluyandi

Native Title Claim

SC98/1

Yandruwandha/Yawarrawarrka

Native Title Claim

PL20

27°0'S

27°0'S

Cooper Basin

Permian Edge

Coongie  Lakes (NP)

Cooper

Innamincka (RR)

Innamincka (RR)

PPL 102

PPL 101

PL5

PL18

28°0'S

PPL 7

"

Moomba

PPL 113

PPL 9

28°0'S

PL17

PPL 8	PPL 11

PL15

SC97/4

Dieri Native Title Claim

PL9

Strzelecki (RR)

Basin

29°0'S

29°0'S

Yandruwandha/Yawarrawarrka Petroleum Conjunctive Indigenous Land Use Agreement (SI2006/008)

PLs7,8

SC99/1

Adnyamathanha No.1

SC08/2

Dieri No 2 Native Title Claim

139°30'E

PL2

PL1

0      10     20     30     40 km

NEW SOUTH WALES

MGA Zone 54

Strzelecki (RR)

141°0'E

REG 204218-017

Petroleum Production Licence (PPL)

Targeted areas of interest for

Indigenous Land Use

Agreement (ILUA)

*  Satellite station

"	Processing plant

deep coal and shale

Targeted areas of interest for basin centred gas

Restricted areas

Coongie Ramsar site

National Estate area

Native Title Claims and Determinations

SC97/3:

The Wangkangurru/Yarluyandi

Native Title Claim

SC97/4:

Dieri Native Title Claim

Liquids

SC98/1:

Gas field

Oil field

Pipeline Licence (PL)

Gas

Gas and liquids

Parks with no petroleum exploration access

Yandruwandha/Yawarrawarrka	 	 Native Title Claim

Parks with petroleum exploration access

SC99/1:

Adnyamathanha No.1

SC08/2:

Dieri No 2 Native Title Claim

Roads

Figure 4.2	Project licences, land access and Native Title, Cooper Basin.

TOOL DARA

Moomba-Big  Lake  Typical  Unconventional  Section

ROSE EPSI MTRE

TOOL DARA

PATC

Gross gas column >600m

ROSE EPSI MTRE

PATC

low of 15 tcf to a high of 125 tcf (raw gas)1. The high-side estimate corresponds to a gross total of more than 200 tcf raw recoverable unconventional gas.

Unconventional Hydrocarbon Resource

Targeted

•  Shale Gas (continuous gas accumulation), PPLs 7,8,9,11

•  Basin-centred Gas (continuous gas

accumulation), PPLs 101, 102, 113

•  Deep Coal Seam Gas (continuous gas accumulation), PPLs 7,8,9,11

Target Formations

•  Toolachee Formation (deep CSG and

GRANITE

Figure 4.3	Moomba-Big Lake typical unconventional section

1	Santos’ equity in the Cooper Basin production licences is 66.6% in South Australia and 60.1% in Queensland. Hence, Santos of estimate of the potential range for recoverable raw gas from its licences in the Cooper Basin is, in gross terms, at least 22 tcf to a high

of more than 200 tcf (raw gas). Santos’ November

2012 estimates of its unconventional resources in the Cooper Basin can be accessed from:  www.santos.com/ library/121112_EABU_Cooper_Basin_Unconventional_ Gas_Opportunities_and_Commercialisation.pdf

Nappamerri Trough Structural

Cross-section

Figure 4.4	Nappamerri Trough structural cross-section.

4

Figure 4.5	Cooper Basin JV Unconventional Gas Project, planned wells for 2012. Langmuir 1, Gaschnitz 1 and Van der Waals 1 are planned vertical wells to test the basin-centred gas play in the Nappamerri Trough. A horizontal well to test the shale gas play at Moomba North, and a vertical well to assess the deep coal seam gas play at Moomba is also planned.

basin-centred gas)

•  Roseneath Shale (shale gas)

•  Epsilon Formation (shale and basin-

centred gas)

•  Murteree Shale (shale gas)

•  Patchawarra Formation (deep CSG and

basin-centred gas)

Target Depths

•  Toolachee coals: 2450-3350 m

•  REM: 2200-2700 m

•  Patchawarra Formation: 2750-3950 m

•  Patchawarra Formation coals:

2850-3500 m

Target Thickness

•  Toolachee coals: Up to 25 m

•  REM: 120 m - >180 m

•  Patchawarra Formation: 300 m

•  Patchawarra Formation coals: Up to 37 m

Exploration Results To End October 2012

•  Moomba 185 shale core analyses are commensurate with US producing shale gas basins.

•  Flowed gas to surface at 100,000 scf/ day from a fracture stimulated deep Patchawarra Formation coal (2745 m) in Moomba 77.

•  A 3 stage fracture stimulation of the Moomba 191 Roseneath-Epsilon- Murteree (REM) section resulted in a stabilised flow rate of 2.6 mmscf/d dry gas. The well has been connected to

the Moomba processing facilities. Santos

estimate 3-6 bcf recoverable gas per

vertical well.

2C Resource Estimate (Pj, End 2010)

•  REM shale: ~ 1026 (Santos share 684)

•  Tight sands and mixed lithology: ~ 1613 (Santos share 1075)

•  Deep Coal: ~ 879 (Santos share 586)

•  TOTAL: ~ 3518 (Santos share 2345)

Environment And Land Access

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats

and risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be approved for compliance before a regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

Forward Plan

2012-2013: (See Figure 4.5)

•  6 x vertical exploration wells to test the basin-centred gas play in the Nappamerri Trough

•  1 x dedicated shale horizontal drilling trial at Moomba North

•  1 x vertical well at Moomba to assess the

deep coal seam gas play

•  Extensive fracturing trials beyond levels currently tested in the Cooper

2013+:

•  Multiple horizontal and vertical appraisal wells

•  Targeting initial shale reserve bookings in 2013 and initial production in 2015. Production now brought forward with connection of moomba 191 to the Moomba processing facilities and first production to sales expected in october

2012.

Contact Details

Name:	Colin Cruikshank – General Manager – Development and Technical Services

Phone:	+61 8 8116 5000

Website:  www.santos.com

4.1.2 Nappamerri Trough Project

Licence And Licence Holders

PEL 218

Beach Energy (90%)*

Adelaide Energy (10%), subsidiary of Beach

Energy

*Denotes operator

Location

Cooper Basin, South Australia (Figure 4.6).

Project Summary

Beach Energy drilled the first purpose drilled vertical shale gas evaluation wells in PEL 218 the Cooper Basin, South Australia (Figure 4.6). Encounter 1 (2010) and Holdfast 1 (2011)

were specifically located off structure, and both wells encountered evidence of extensive gas saturation through the Permian section (see figures 4.7 and 4.8).

Holdfast 1 flowed gas at up to 2 mmscf/d after a seven stage fracture stimulation of the Early Permian succession. Encounter 1 was fracture stimulated on the Patchawarra interval in April 2012, flowing up to 0.75 mmscf/d, and on the REM package (5 stages) in June 2012, flowing 1.3 mmscf/d. Beach has stated that gas desorption from

26°0'S

139°30'E

QUEENSLAND

4

26°0'S

141°0'E

Wangkangurru/Yarluyandi Petroleum Conjunctive Indigenous Land Use Agreement (SI2011/023)

SC97/3

The Wangkangurru/Yarluyandi

Native Title Claim

SC98/1

Yandruwandha/Yawarrawarrka

Native Title Claim

PL20

27°0'S

27°0'S

Cooper Basin

Permian Edge

Coongie  Lakes (NP)

Cooper

Innamincka (RR)

PEL 218

PL5

PL18

28°0'S

28°0'S

Moomba "

PL17

PL15

SC97/4

Dieri Native Title Claim

PL9

Strzelecki (RR)

Basin

29°0'S

29°0'S

Yandruwandha/Yawarrawarrka Petroleum Conjunctive Indigenous Land Use Agreement (SI2006/008)

PLs7,8

SC99/1

Adnyamathanha No.1

SC08/2

Dieri No 2 Native Title Claim

139°30'E

PL2

PL1

0      10     20     30     40 km

NEW SOUTH WALES

MGA Zone 54

Strzelecki (RR)

141°0'E

REG 204218-009

Selected Petroleum tenements

Petroleum Exploration

Indigenous Land Use

Agreement (ILUA)

*  Satellite station

" 	Processing plant

Licence 218

Restricted areas

Coongie Ramsar site

National Estate area

Parks with no petroleum exploration access

Native Title Claims and Determinations

SC97/3:

The Wangkangurru/Yarluyandi

Native Title Claim

SC97/4:

Dieri Native Title Claim

SC98/1:

Gas field

Oil field

Pipeline Licence (PL)

Gas

Gas and liquids

Parks with petroleum exploration access

Yandruwandha/Yawarrawarrka

Liquids

Native Title Claim

SC99/1:

Adnyamathanha No.1

SC08/2:

Dieri No 2 Native Title Claim

Roads

Figure. 4.6	PEL 218 location, land access and Native Title.

Triassic sediments Non-prospective No aquifers

Toolachee Formation

Daralingie Formation

Gas Target Zone

Roseneath Shale

Epsilon Formation	 	 Murteree Shale

Patchawarra Formation

700 m Gas

Saturated

Potential

600 m Gas

wells. However Beach has since adopted the new SPE Petroleum Resources Management System guidelines for booking unconventional resource volumes. The new

2C resource estimate for PEL 218 is 1.3 tcf (based on Holdfast 1 and Encounter 1 only, comprising 28 km2 around each well for sandstones and 41 km2 around each well for shales).

Unconventional Hydrocarbon Resource

Targeted

Shale Gas (continuous gas accumulation)

•  Basin-centred Gas (continuous gas

accumulation)

•  Discrete Tight Gas (within closure)

Tirrawarra Sandstone

Sandstone	Shale	Coal

Figure 4.7	Gas target zone, Nappamerri Trough.

the shales cored in the two wells suggests almost 100 tcf of shale gas in place in

PEL 218.

Cooper Basin

The presence of a basin-centred gas accumulation (BCGA) in the Nappamerri Trough has been suspected for over two decades. Beach has stated that the Encounter and Holdfast wells confirm the presence of gas in sands outside of structural closure and estimate that gas in place potentially exceeds 200 tcf in sands across PEL 218. Beach is accelerating exploration

in PEL 218. Recent wells Moonta 1 and Streaky 1 encountered gas saturation in the target Permian section and fracture stimulations commenced in October 2012.

These wells are the first two of between 5 and

8 vertical wells planned for PEL 218 in 2012-

2013 (Figure 4.9). The first horizontal well in South Australia is expected to be Holdfast 2, currently planned for Q4 2012.

Beach booked an initial gross contingent resource of 2 tcf of sales gas constrained

to a 10 x 10 km area around the Encounter

1 and Holdfast 1 shale gas evaluation

Target Formations

•  Toolachee Formation (sand)

•  Daralingie Formation (sand)

•  Roseneath Shale (shale)1

•  Epsilon Formation (sand and shale) 2

•  Murteree Shale (shale)1

•  Patchawarra Formation (sand)1

Target Depths

2600 – 4000 m

Target Thickness

1400 m

Exploration Results To End October 2012

•  Encounter 1 drilled late 2010

•  Holdfast 1 drilled early 2011

•  Fracture stimulated Holdfast 1 with

7 stages (Patchawarra x 1 stage, Murteree x 1 stage, Epsilon x 3 stages and Roseneath x 2 stages)

•  Flow tested Holdfast 1 with maximum rates of 2 mmscf/d

•  Gas in place in PEL 218 estimated at up to 300 tcf, comprising almost 100 tcf gas in shales (estimated from gas desorption analyses of shales cored in Encounter 1 and Holdfast 1) and in excess of 200 tcf gas in sands.

2	Fracture stimulated and flow tested in Holdfast-1 and

Encounter 1.

4

Figure 4.8	Schematic section through the Nappamerri Trough showing the extent of the unconventional gas plays and

planned (dashed) and drilled (solid line) unconventional wells.

•  Moonta 1 encountered gas saturation over more than 1,000m of the Permian target zone (Toolachee Formation to base of the Patchawarra Formation).

•  Fracture stimulation of Moonta 1, focusing primarily on the Patchawarra Formation, has commenced (Patchawarra Formation – nine stages, Murteree Shale – one stage)

Upside Potential

The Encounter 1 and Holdfast 1 wells were terminated in the upper Patchawarra Formation. Potential for a further 400-600m of gas saturation in the Patchawarra Formation is being tested with the current drilling programme.

2C Resource Estimate (August 2012)

Beach has adopted the new SPE Petroleum Resources Management System guidelines for booking unconventional resource volumes. The new 2C resource estimate for PEL 218 is 1.3 tcf (based on Holdfast 1 and Encounter 1 only, comprising 28 km2 around

each well for sandstones and 41 km2 around each well for shales).

The gas-in-place estimate is unchanged at

300+ tcf for PEL 218.

Environment And Land Access

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in

consultation with stakeholders and relevant government agencies, addressing potential threats and risks to the environment, and

the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed

Figure 4.9      PEL 218, Nappamerri Trough, South Australia: Existing vertical shale gas evaluation wells are Encounter-1 (2010), Holdfast-1 (2011), Moonta 1 and Streaky 1. Horizontal wells at Encounter and Holdfast are planned to assess gas deliverability from shales.

through an open, consultative process and based on information in the EIR, must then be approved for compliance before a regulated activity can proceed. See

Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

The Land Access Agreement for PEL 218, pursuant to section 31 of the Native Title

Act 1993 (Cth) can be downloaded via the following link: www.petroleum.dmitre.sa.gov. au/environment/native_title,_aboriginal_ lands,_iluas/current_native_title_agreements

Forward Plan

2012 – Delineate basin-centred gas and tight gas concept in PEL 218 with vertical well drilling, stimulation and flow test program. Commence horizontal well program to estimate deliverability from shales.

2013 – Continue vertical and horizontal well drilling, fracture stimulation and flow testing. Drill appraisal wells to test deliverability potential ‘production pilot concept’.

2014 – Continue appraisal of production potential through drilling, stimulation and flow testing of production pilot wells.

Contact Details

Name:	Mike Dodd – Exploration and

Development Manager

Phone:	+61 8 8338 2833

Website:  www.beachenergy.com.au

4.1.3 Senex Energy Southern Cooper

Basin Unconventional Gas Project

Licence and Licence Holders

PEL 516

Senex Energy Limited (100%)* PEL 115

Senex Energy Limited (55%)* Orca Energy Limited (20%) Others (25%)

Location

Cooper Basin, South Australia (Figure 4.10).

Project Summary

Senex Energy Limited acquired PEL 516 (Figure 4.10) following the acquisition of Stuart Petroleum in 2011. Assessment of unconventional gas plays in the licence commenced shortly after with the coring of Toolachee Formation coals and Roseneath and Murteree shales in the Vintage Crop

oil discovery well. Senex reported that the Toolachee Formation coals and REM (Roseneath – Epsilon- Murteree) shale formations exhibited continuous gas shows, as well as the presence of liquids and heavy gases.

A nine metre core in the Toolachee Formation and two nine metre cores in each of the Roseneath and Murteree shales were cut in Vintage Crop 1 for gas desorption analysis and other tests. Senex has reported that preliminary results confirm the presence of liquids rich gas, favourable mineralogy for fracture stimulation, and properties similar to successful North American shale plays.

Senex has accelered unconventional gas exploration in PEL 516 in 2012 drilling three vertical wells to end October,  with results as follows:

•  A multi-zone fracture stimulation of Patchawarra Formation tight sands and the Roseneath and Murteree shales in Sasanof 1 was completed in May 2012. The well produced fracture stimulation fluid and liquids rich gas at a peak rate of >200,000 scf/day.

•  High gas readings with evidence of condensate and heavy gases were

recorded in the Permian section in	4

Talaq 1.

•  Wireline logs and mud logs from Skipton

1 have confirmed material quantities of hydrocarbons outside of structural closure. Skipton 1 intersected more than 75 metres of interpreted net gas pay in the Patchawarra Formation and

164 metres of gas charged Roseneath and Murteree shales.

Senex commenced drilling its fourth operated unconventional gas exploration well, Kingston Rule 1 (PEL 115) in October

2012. A large scale hydraulic fracture stimulation program at Skipton 1, Talaq 1 and Kingston Rule 1 to test gas quality and deliverability is planned to commence in December 2012.

Senex has completed a $155 million equity raising to fund the planned FY2012-2013+ work program. The work includes 10+ vertical

wells targeting unconventional gas in Senex’s southern Cooper Basin permits (primarily PEL

516), following the early success at Sasanof 1, Talaq 1 and Skipton 1.

MHA Petroleum Consultants has estimated

potential gas-in-place in PEL 516 of over

100 tcf.

Unconventional Hydrocarbon Resource

Targeted

•  Shale Oil

•  Shale Gas (continuous gas accumulation)

•  Basin-centred Gas (continuous gas

accumulation)

•  Deep Coal Seam Gas (continuous gas accumulation)

Target Formations (Figures 4.11 and 4.12)

•  Toolachee Formation (deep CSG)

•  Roseneath Shale (shale)

•  Epsilon Formation (shale)

•  Murteree Shale (shale)

•  Patchawarra Formation (basin centred gas)

26°0'S

139°30'E

QUEENSLAND

141°0'E

26°0'S

Wangkangurru/Yarluyandi Petroleum  Conjunctive Indigenous Land Use Agreement (SI2011/023)

SC97/3

The Wangkangurru/Yarluyandi

Native Title Claim

SC98/1

Yandruwandha/Yawarrawarrka

Native Title Claim

PL20

27°0'S

27°0'S

Cooper Basin

Permian Edge

Coongie  Lakes (NP)

Keleary

Cooper

Innamincka  (RR)

Basin

PL5

PL18

PEL 516

Flax East 1

28°0'S

28°0'S

Innamincka  (RR)

Moomba "

PL17

SC97/4

Dieri Native Title Claim

Strzelecki (RR)

PEL 516

PEL 115

PL15

PL9

PLs7,8

Dullingari

Moomba South

Bi

TaonoCtlaencnnhtareael Daralingie g Lake

29°0'S

29°0'S

Yandruwandha/Yawarrawarrka Petroleum  Conjunctive Indigenous Land Use Agreement (SI2006/008)

SC99/1

Adnyamathanha No.1

SC08/2

Dieri No 2 Native Title Claim

139°30'E

PL2

PL1

0      10     20     30     40 km

NEW SOUTH WALES

MGA Zone 54

Strzelecki (RR)

141°0'E

REG 204218-010

Selected Petroleum tenements

Indigenous Land Use

*  Satellite station

Petroleum Exploration	 	  Agreement (ILUA)

" 	Processing plant

Licences 115 and 516

Restricted areas

Coongie Ramsar site

National Estate area

Parks with no petroleum exploration access

Native Title Claims and Determinations

SC97/3:

The Wangkangurru/Yarluyandi

Native Title Claim

SC97/4:

Dieri Native Title Claim

SC98/1:

Gas field

Oil field

Pipeline Licence (PL)

Gas

Gas and liquids

Parks with petroleum exploration access

Yandruwandha/Yawarrawarrka

Liquids

Native Title Claim

SC99/1:

Adnyamathanha No.1

SC08/2:

Dieri No 2 Native Title Claim

Roads

Figure 4.10	PELs 115 and 516 location, land access and Native Title, Cooper Basin.

Teritiary

-2000 m

Cretaceous

Jurassic

Sasanof-1

shales in Sasanof 1 was completed in	4

May 2012. The well has produced fracture

stimulation fluid and liquids rich gas at

a peak rate of >200,000 scf/day. Talaq

1 encountered high gas readings with evidence of condensate and heavy gases in the Permian section and Skipton

Eromanga Aquifer

Triassic

Toolachee Formation

Gas Target Zone

Permian

Daralingie Formation

-900 m

Roseneath Shale Epsilon Formation Murtaree Shale

Patchawarra Formation

Tirrawarra Sandstone

Sandstone	Shale	Coal

Figure 4.11	Gas target zone for PEL 516.

Target Depths

2800-3200 m

Target Thickness

600-900m (Permian succession)

Exploration Results To End October 2012

Vintage Crop 1 cut a nine metre core in the

Toolachee Fm and two nine metre cores in each of the Roseneath and Murteree shales. Wet gas shows were encountered through the coring interval. Senex reported that preliminary results indicate the potential resource in PEL 516 is in the wet gas window, containing liquids and heavy gases with a low carbon dioxide content.

Assessment of the unconventional gas potential of PELs 516 and 115 is progressing with the drilling of the Sasanof 1, Talaq 1 and Skipton 1 vertical wells, and the Kingston

Rule 1 well in progress at end October

2012 (Figure 4.13) . A multi-zone fracture stimulation of Patchawarra Formation tight sands and the Roseneath and Murteree

1 intersected more than 75 metres of interpreted net gas pay in the Patchawarra Formation and 164 metres of gas charged Roseneath and Murteree shales.

Resource Potential

>100 tcf gas-in-place

Environment And Land Access

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats

and risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be approved for compliance before a regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

The land access agreements for PELs 516 and

115, pursuant to section 31 of the Native Title Act 1993 (Cth), can be downloaded via the following link: www.petroleum.dmitre.sa.gov. au/environment/native_title,_aboriginal_ lands,_iluas/current_native_title_agreements

Forward Plan

2012-2013+:

•  Fracture stimulation of Talaq 1, Skipton 1 and Kingston Rule 1 to test deliverability and gas composition.

•  Book material contingent resource

•  10+ vertical wells targeting unconventional gas in Senex’s southern Cooper Basin permits (primarily PEL 516)

•  Focus on flow testing (limited coring)

•  Pilot program to follow a successful appraisal program

Contact Details

Name:	Ian Davies – Managing Director

Phone:	+61 7 3837 9900

Website:  www.senexenergy.com.au

Figure 4.12	Regional N-S log correlation across the Cooper Basin, South Australia, highlighting the shallower occurrence of the prospective Permian succession in the Tenappera Trough/Mettika Embayment compared with the Nappamerri Trough. Senex’s planned Skipton-1 and Talaq-1 unconventional wells will test the Mettika Embayment. The relatively shallow depths mean lower temperatures are encountered, and there is no need for specialised casing or drilling materials.

4

Figure 4.13	Senex Energy Limited Unconventional Gas Project, planned wells for 2012.

4.1.4 Beach Energy JV Coal Seam Gas and Shale Gas Project Licence and Licence Holders:

PEL 94

Beach Energy Limited (50%)* Strike Energy Limited (35%) Senex Energy Limited (15%) PEL 95

Beach Energy Limited (50%)*) Strike Energy Limited (50%)

*Denotes operator

Location

Cooper Basin, South Australia (Figure 4.14)

Project Summary

PELs 94 and 95 were originally granted to Beach Petroleum NL (50%) and Magellan Petroleum (NT) Pty Ltd (50%) in 2001

(Figure 4.14). Beach Energy has retained a

50% interest in the licences and is operator of both licences.

Marsden 1 was drilled in the deepest part of Battunga Trough approximately 6 km southeast of the Battunga 1 well (Santos

Ltd, 1987) in 2012. A 20 m Patchawarra coal seam with mud gas readings up to 1835 units was intersected in Battunga 1. A 20 m Toolachee coal seam was also intersected

in Battunga 1. However the coal is less thermally mature peaking at around 20 units in the main seam and up to 500 units in thin overlying coal seams.

Marsden 1 (2012) was drilled specifically to test the coal seam gas potential of the Permian coals and the shale gas potential

26°0'S

26°0'S

139°30'E

QUEENSLAND

141°0'E

Wangkangurru/Yarluyandi Petroleum  Conjunctive Indigenous Land Use Agreement (SI2011/023)

Yandruwandha/Yawarrawarrka Petroleum  Conjunctive Indigenous Land Use Agreement (SI2006/008)

SC97/3

The Wangkangurru/Yarluyandi

Native Title Claim

SC98/1

Yandruwandha/Yawarrawarrka

Native Title Claim

PL20

27°0'S

27°0'S

Cooper Basin

Permian Edge

Coongie  Lakes (NP)

Keleary

Cooper

Innamincka (RR)

Basin

PL5

PL18

Flax East 1

28°0'S

28°0'S

Innamincka (RR)

Moomba "

PL17

SC97/4

Dieri Native Title Claim

PL15

PL9

Dullingari

Moomba South

Bi

TaonoCtlaencnnhtareael Daralingie g Lake

Strzelecki (RR)

29°0'S

29°0'S

PEL 94

PEL 95

PLs7,8

SC99/1

Adnyamathanha  No.1

SC08/2

Dieri No 2 Native Title Claim

139°30'E

PL2

PL1

0      10     20     30     40 km

NEW SOUTH WALES

MGA Zone 54

Strzelecki (RR)

141°0'E

REG 204218-036

Selected Petroleum tenements

Indigenous Land Use

*  Satellite station

Petroleum Exploration	 	  Agreement (ILUA)

"	Processing plant

Licences 94 and 95

Restricted areas

Coongie Ramsar site

National Estate area

Native Title Claims and Determinations

SC97/3:

The Wangkangurru/Yarluyandi

Native Title Claim

SC97/4:

Gas field

Oil field

Pipeline Licence (PL)

Parks with no petroleum

Dieri Native Title Claim	 	  Gas

exploration access

Parks with petroleum exploration access

SC98/1: Yandruwandha/Yawarrawarrka Native Title Claim

SC99/1: Adnyamathanha No.1

SC08/2:

Dieri No 2 Native Title Claim

Gas and liquids

Liquids

Roads

Figure 4.14	PELs 94 and 95 location, land access and Native Title, Cooper Basin.

Figure 4.15	Gas target zone for Marsden 1 (based on

nearby Battunga 1 well).

of the Murteree Shale in PEL 95 (Figures

4.15, 4.16 and 4.17). Marsden 1 intersected a thick coal interval and gas readings

were elevated throughout the Permian section. Early core analysis results indicate high gas contents. The well was cased and suspended for further evaluation.

Davenport 1 (2012) was drilled in the deepest part of the Milpera Trough to evaluate the unconventional gas potential of Permian coals and shales in PEL 94 (Figure 4.18). The well was cased and suspended for future evaluation after intersecting 110m of net

coal including one seam in the Patchawarra with over 45m of coal. Coal cores are currently being analysed for gas content and other physical parameters.

Unconventional Hydrocarbon Resource

Targeted

•  Shale Gas (continuous gas accumulation)

•  Deep Coal Seam Gas (continuous gas accumulation)

Target Formations	4

•  Toolachee Formation (deep CSG), PEL 95

•  Murteree Shale (shale), PEL 95

•  Patchawarra Formation (deep CSG), PELs 94 and 95

Target Depths

•  PEL 94, Patchawarra Formation:

1990-2175 m

•  PEL 95, Toolachee Formation:

1870-1980 m

•  PEL 95, Patchawarra Formation:

2450-2550 m

•  PEL 95, Murteree Shale: 2160-2225 m

Target Thickness

•  PEL 94, Patchawarra Formation: 5-25 m

•  PEL 95, Toolachee Formation: 15-25 m

•  PEL 95, Patchawarra Formation: 15-25 m

•  PEL 95, Murteree Shale: 65 m

Target Formations

•  Toolachee Formation (deep CSG), PEL 95

•  Murteree Shale (shale), PEL 95

•  Patchawarra Formation (deep CSG), PELs 94 and 95

Target Depths

•  PEL 94, Patchawarra Formation:

1990-2175 m

•  PEL 95, Toolachee Formation:

1870-1980 m

•  PEL 95, Patchawarra Formation:

2450-2550 m

•  PEL 95, Murteree Shale: 2160-2225 m

Target Thickness

•  PEL 94, Patchawarra Formation: 5-25 m

•  PEL 95, Toolachee Formation: 15-25 m

•  PEL 95, Patchawarra Formation: 15-25 m

•  PEL 95, Murteree Shale: 65 m

Exploration Results To End July 2012

See Project Summary

Resource Potential

No resource estimation available at this stage.

Figure 4.16	Log correlation showing rapid thinning of the Patchawarra Formation on the margins of the Battunga Trough, Cooper Basin, South Australia. The target Permian coal seams are best developed in the Battunga Trough. In contrast the REM shales are a relatively consistent thickness.

Environment and Land Access Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or drilling) is

a separate process that is subject to a separate, rigorous assessment and consultation process, so

that operations are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats and

Figure 4.17	Map showing depth to the top of the Patchawarra Formation and the location of Marsden 1 in the deepest

part of the Battunga Trough.

4

Figure 4.18	Map showing depth to the top of the Patchawarra Formation and the location of Davenport1 in the deepest

part of the Milpera Trough.

risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be approved for compliance before a regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

Land access agreements for PELs 94 and

95, pursuant to section 31 of the Native Title

Act 1993 (Cth), can be downloaded via the following link: www.petroleum.dmitre.sa.gov. au/environment/native_title,_aboriginal_ lands,_iluas/current_native_title_agreements

Forward Plan

A decision regarding the forward plan will be made when core analyses are completed.

Contact Details

Name:	Mike Dodd – Exploration and

Development Manager

Phone:	+61 8 8338 2833

Website:  www.beachenergy.com.au

4.1.5 Southern Cooper Coal Seam

Gas Project

Licence

PEL 96

Licence Holders

Strike Energy Limited (66.67% and Operator) AGL (33.33%)

Location

Cooper Basin, South Australia (Figure 4.19)

Project Summary

PEL 96 was granted to Strike Oil Limited (now

Strike Energy) and AGL in 2009 (Figure. 4.19).

Forge-1, drilled in 2010, was the first exploration well drilled to test the coal seam gas potential of the Permian coals in PEL 96 (Figures. 4.20 and 4.21). Drilling achieved

a partial test of the proposed drill target. The full 224 m section of Permian coal measures was not drilled due to suspension of drilling for operational reasons. The target Permian coal measures were penetrated immediately beneath the base Eromanga Basin unconformity and were consequently oxidised. Biogenic gas contents in the order of 1.1 m3/t (40 scf/ton) were recorded in

the coal cores submitted for gas desorption

analysis.

Follow up drilling in PEL 96 is not planned until

2014 (Figure 4.22).

Unconventional Hydrocarbon Resource

Targeted

•  Shale Gas (continuous gas accumulation)

•  Coal Seam Gas (continuous gas accumulation)

Target Formations

•  Toolachee Formation (CSG)

•  Roseneath Shale (shale)

•  Epsilon Formation (shale, CSG)

•  Murteree Shale (shale)

•  Patchawarra Formation (CSG)

Target Depths

1000-2500 m

Target Thickness

•  Coal Seam Gas: 5–75 m net coal thickness

•  Shale Gas: 30–60 m net shale thickness

Exploration Results To End 2011

The Forge 1 well penetrated an aggregate thickness of 21 m of coal from 1,281m in the upper part of the Permian section. The full sequence, including the primary coal target in the Patchawarra Formation, was not drilled due to operational reasons. Biogenic gas contents in the order of 1.1 m3/t (40 scf/ton) were recorded.

Resource Potential

•  CSG: Prospective Resource Estimate 3.7

– 9.3 tcf (Gross)

•  Shale Gas: Prospective Resource

Estimate 0.36 – 0.95 tcf (Gross)

Environment And Land Access

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats

and risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be approved for compliance before a regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

26°0'S

139°30'E

QUEENSLAND

4

26°0'S

141°0'E

Wangkangurru/Yarluyandi Petroleum Conjunctive Indigenous Land Use Agreement (SI2011/023)

SC97/3

The Wangkangurru/Yarluyandi

Native Title Claim

SC98/1

Yandruwandha/Yawarrawarrka

Native Title Claim

PL20

27°0'S

27°0'S

Cooper Basin

Permian Edge

Coongie Lakes (NP)

Cooper

Innamincka (RR)

PL5

PL18

28°0'S

28°0'S

Moomba "

PL17

PL15

SC97/4

Dieri Native Title Claim

PL9

Strzelecki (RR)

Basin

29°0'S

Yandruwandha/Yawarrawarrka Petroleum Conjunctive Indigenous Land Use Agreement (SI2006/008)

PLs7,8

29°0'S

NEW SOUTH WALES

PEL 96

SC99/1

Adnyamathanha No.1

SC08/2

Dieri No 2 Native Title Claim

139°30'E

PL2

PL1

0      10     20     30     40 km

MGA Zone 54

Strzelecki (RR)

141°0'E

REG 204218-011

Selected Petroleum tenements

Petroleum Exploration

Indigenous Land Use

Agreement (ILUA)

*  Satellite station

" 	Processing plant

Licence 96

Restricted areas

Coongie Ramsar site

National Estate area

Parks with no petroleum exploration access

Native Title Claims and Determinations

SC97/3:

The Wangkangurru/Yarluyandi

Native Title Claim

SC97/4:

Dieri Native Title Claim

SC98/1:

Gas field

Oil field

Pipeline Licence (PL)

Gas

Gas and liquids

Parks with petroleum exploration access

Yandruwandha/Yawarrawarrka

Liquids

Native Title Claim

SC99/1:

Adnyamathanha No.1

SC08/2:

Dieri No 2 Native Title Claim

Roads

Figure 4.19	PEL 96 location, land access and Native Title, Cooper Basin.

Basin

Conventional

Unconventional

Depth

Eromanga

Oil	800-1500 m

Cooper

Natural Gas

Oil

Toolachee Coal

Daralingie Fm

Roseneath Shale Epsilon Coal Murteree Shale

Patchawarra Coal

CSG

Shale Gas

Tight Gas

1000-2500 m

Figure 4.20	Gas target zone for PEL 96.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

The Land Access Agreement for PEL 96, pursuant to section 31 of the Native Title Act

1993 (Cth), can be downloaded via the following link: www.petroleum.dmitre.sa.gov. au/environment/native_title,_aboriginal_ lands,_iluas/current_native_title_agreements

Forward Plan

•  2012: Evaluation of target prospectivity.

•  2013: Seismic survey.

•  2014: Drilling of 1 – 2 deep wells.

Contact Details

Name:	Don Poynton – Managing of

Exploration and Environment

Phone:	+61 8 6103 0999

Website:  www.strikeenergy.com.au

4

Figure 4.21	Log correlation across the southern Cooper Basin, South Australia. The target Permian coal seams and REM

shales are generally shallower than 2000 m in PEL 96.

Figure 4.22	Map showing depth to the top of the Permian succession in licences in which Strike Energy has an interest. The

top of the Permian succession in PEL 96 is less than 1500 m over the entire licence. No exploration wells are planned for PEL

96 in 2012.

4.1.6 Otway Basin Gas Project

Licences and Licence Holders

PEL 186

Beach Energy (66.67%) Cooper Energy (33.33%)*

PEL 255

Beach Energy (100%)*

PEL 494

Beach Energy (100%)*

PEL 495

Cooper Energy (60%)* Beach Energy (35%)

*Denotes operator

Location

Otway Basin, South Australia (Figure 4.23)

Project Summary

Within the South Australian portion of the Otway Basin, the Casterton Formation is considered to have highest prospectivity for unconventional oil and gas plays (Figure 4.24).

The Casterton Formation contains shales

that are the source rocks of the commercial gas accumulations in the Penola Trough

and has also generated oil, as evidenced

by the widespread oil flows/recoveries

and shows, particularly on the flanks of the basin. The formation is widespread but has only been penetrated in about ten wells on the northern and eastern flanks of the

Basin, where the Casterton is not sufficiently thermally mature to present a viable shale gas play.

The existing well control indicates an average prospective shale sequence in

the Casterton of about 85 m. However, the formation is interpreted to reach thicknesses in excess of 300 m in the undrilled troughs, where it is thermally mature to overmature and may be overpressured (the deepest Casterton penetration to date is at approximately 2,500 m).

On a regional basis, the Casterton Formation is generally a moderately rich, gas prone

source rock, with total organic carbon (TOC) averaging approximately 2.5%. However the source richness of the formation can vary widely over relatively short distances and in some portions of the basin it has excellent

oil source capacity, with TOCs of up to 20% observed in western Victoria. Although these intervals are thin and not widespread, where they are present and thermally mature

they are capable of generating significant

volumes of oil.

The Casterton Formation was deposited in a cold climate, lacustrine environment and contains predominately siliceous shales containing minor amounts of siderite. As such they have similar mineralogy to the REM shales in the Nappamerri Trough of the Cooper Basin and some producing shales in North America.

Shales and low permeability sands within the lower Pretty Hill Formation may also present viable unconventional plays, where they

are thermally mature and the Casterton Formation may be too deep to present an economic target.

Unconventional Hydrocarbon Resource

Targeted

•  Shale Gas

•  Basin-centred Gas

Target Formations

Casterton Formation and lower Pretty Hill

Formation

Target Depths

2,600 – 4,000 m

Target Thickness

Up to 300 m

Exploration Results To End July 2012

Most of the Casterton well penetrations to date have been in western Victoria where

it is present at relatively shallow depths. The only definitive Casterton penetration in South Australia to date is at Sawpit 1, which was drilled on the northern flank of the Penola Trough and penetrated 61 m of Casterton Formation at a depth of about 2,500 m.

140°0'E

Kingston

Padthaway

Talapar  (CP) PRESWA

Grass Tree (CP)

141°0'E	4

Rosetown, Grave Reserve

Kingston "

Butcher Gap (CP)

Mount Scott (CP)	Fairview (CP)

Mullinger

Swamp  (CP)

37°0'S

Vivigani Ardune (CP)   "

37°0'S

Naracoorte "

Kungari (CP)

PEL 186

Lucindale

Naracoorte

Caves  (NP)

PL13

Guichen Bay (CP)

Robe

Woakwine (CP)

Big Heath  (CP)

Hacks Lagoon  (CP)

Robe Trough 	"

Lake Hawdon

Mary  Seymour  (CP)

Bool Lagoon

(GR)

!

Lake Robe (GR)

Little Dip (CP)

Lake St Clair (CP)

South (CP)

186

St Clair

Penola

Reedy Creek  (CP) Trough

PEL 255

Penola (CP)

Glen  Roy (CP)

PEL 495

Penola

PL16

Trough

Furner  (CP)

Calectasia 	"

VICTORIA

(CP)

Beachport (CP)

Beachport "

Penguin Island  (CP)

Belt

Hill  (CP)

PEL 494

PL3

PL19

Lake Frome (CP)

"

Millicent

PL4

Otway

Canunda (NP)

Tantanoola

Caves (CP)

Telford Scrub (CP) Gower  (CP)

Basin

Canunda (NP)

Mount Gambier "

Lower Limestone

Bucks Lake (GR) Coast PRESWA

38°0'S

0	10	20	30	40 km

MGA Zone 54

Carpenter Rocks (CP) Nene  Valley (CP)

Douglas Point (CP)

38°0'S

Penambol (CP)

Lower Glenelg River (CP)

Ewens Ponds (CP)

140°0'E

Dingley Dell (CP)

Piccaninnie Ponds (CP)

141°0'E

REG 204218-038

Selected petroleum tenements

Petroleum Exploration

Licences 186, 255, 494, and 495

Restricted areas

Aboriginal Lands

Prescribed Wells Areas (PRESWA) Bool and Hacks Lagoons Ramsar site

Parks with petroleum exploration access

Parks with no petroleum exploration access

Gas pipeline Gas field Coalfield

Oil field

Structural elements

Hinge

Casterton Formation

Play Fairway

Highway

Roads

Railway

Figure 4.23	PELs 186, 255, 494 and 495, land access and Native Title, Otway Basin.

Stratigraphic Column, Otway Basin

approved for compliance before a

| Teritiary |  | Heytesbury Group |
| --- | --- | --- |
|  |  | Nirranda Group |
|  |  | Nirranda Group |
| Cretaceous | Early Late | Sherbrook Group |
|  |  | Otway Supergroup |
| Jurassic |  |  |
| Palaeozoic |  | Kanmantoo Group |

Chronostratigraphy

Age

Otway Supergroup

Alb

Apt

Bar

Hau

Val

Ber

Tith

Kim

Rock Unit

Eumeralla

Formation

Windermere/Katnook  Sandstone

Lair Formation

Crayfish Group

Pretty Hill Fmn

Pretty Hill

Sandstone

Upper Sandpit Shale Mbr

Sandpit Sandstone Mbr

Lower Sandpit Shale Mbr

Upper Sandpit Shale Mbr

v    v    v    v    v    v    v    v    v    v    v    v    v    v

regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

Forward Plan

The Sawpit-2 exploration well (PEL 495) is

expected to commence drilling in Q4 2012.

Figure 4.24	Stratigraphic Column, Otway Basin.

Seismic interpretation indicates that the Casterton thickens considerably into the troughs where it is interpreted reaches thicknesses in excess of 300 m and depths in excess of 4,000 m.

Resource Potential

The potential Gas In Place in the Casterton play is assessed to be in the range 17 - 58 tcf. Approximately 75% of the play lies within the South Australian portion of the basin.

Environment And Land Access

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats

and risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be

The well will test a conventional oil target and will be deepened to the Casterton Formation to acquire core to obtain further information in relation to its capacity to host unconventional resources.

3D seismic was acquired in April 2012 in PEL

186 (Robe Trough) with the aim of maturing a location for drilling in 2013.

Contact Details

Name:	Michael Dodd

(Beach Energy Limited) Phone:	+618 8338 2833

Website:  www.beachenergy.com.au

Name:	Steve Twartz

(Cooper Energy Limited) Phone:	+618 9489 3777

Website:  www.cooperenergy.com.au

4.2 COAL GASIFICATION PROJECTS, SOUTH AUSTRALIA	4

Marla

)"

EL

4513

!(

")  Oodnadatta

(!

(!	EL 4512

Moomba ")

EL 4511

Arckaringa     !(

!(

Coober Pedy

")")

!( CAIRN HILL

(!

Snafell     (!

!(

CHALLENGER !(

PROMINENT HILL

(!

Hawks  Nest

((

!( OLYMPIC DAM

(

Four Mile !(!(! BEVERLEY

!((

AMBROSIA A!tacama

(!(!( Sonoran

")

Tarcoola

Reliance !(!! LEIGH CREEK BELTANA

JACINTH

Typoon

!( Tripitaka

Tunkillia !(

Ceduna

!(

Carrapateena

Portia

!(

Junction

( (

Kalkaroo !( !Dam

HONEYMOON

)" 	Crocker Well !(

White Dam  North

Ê

Thevenard

Port

WHITE DAM !(!(!(

!(

Paris !(

Menninnie Dam

!(

)"  Augusta

Mutooroo

!(

!(

Poochera

WILLCHERRY HILL !(

hyalla

Mutooroo

W (!	Ê ÊPort Bonython

!( Razorback

Warramboo !(

IRON CHIEFTAIN!(

HEP

!(

Port Pirie

Ê")

Port Pirie

Current as at 11 September 2012

Bramfield  !(

!(

MIDDLEBACK RANGES  Samphire

!( Bungalow

!(

Mining projects

!( 	Operating mine

!( 	Mineral development project

WILGERUP

!( Fusion

PEL 120

Port Lincoln

Hillside !(

Port

Exploration tenements

Mineral Exploration Licence (EL)

Gum Flat !!( Ê)"  Port Lincoln

(

Uley

Adelaide

Ê

ADELAIDE ")

!( Bird in Hand

!(

MINDARIE

Petroleum Exploration Licence (PEL)

Coalfield

Infrastructure ")        Locality Ê  Port

Highway

Major road

Railway – standard gauge Railway – non standard gauge Gas pipeline

Liquids pipeline

132 000 volt powerline

KANMANTOO !(

!(

ANGAS

PEL 127

EL 4576

0 	100 	200 	300 Kilometres

Datum GDA 94 – Projection South Australia Lambert

")

275 000 volt powerline

Mount Gambier

Figure 4.25	Map of South Australia showing oil and gas infrastructure, powerlines, mining projects and the location of coal

gasification projects.

4.2.1 Arckaringa CTL and Power

Project

Licences and Licence Holders

ELs 4511, 4512, 4513

CNOOC New Energy International (Australia) Pty Ltd.*(51%)

Arckaringa Energy Pty. Ltd. - fully owned

subsidiary of Altona Energy Plc (49%)

*Denotes operator

Location

Arckaringa Basin, South Australia (Figure 4.26)

Project Summary

The Arckaringa Coalfield, comprising four deposits of sub-bituminous coal in the northern Arckaringa Basin (Figure 4.26), was discovered in 1980 by Meekatharra Minerals.

Systematic exploration of the licences was carried out in the early 1980s at which time

a resource of 7,850 Mt was declared. Altona

Energy Plc (Altona) acquired the coal

assets in 2005 and undertook a series of pre- feasibility studies between 2006 and 2008. These studies included a drilling program with associated testing and analyses, pre- feasibility work on CTL and Power Plant options, optimisation work on mine costs, fuel and electricity market studies, and construction of a Project Economic Model.

Completion of the field exploration program in 2008 resulted in a JORC Resource Estimate of 1.287 billion tonnes for part of the Wintinna deposit which is covered by EL 4512 (Figures

4.26, 4.27 and 4.28). The outcome of the pre- feasibility studies was the establishment of a base case scenario comprising a 10 mtpa mine supplying coal to a CTL plant with an output of 10 mmbbls per annum liquid fuels (mainly ultra clean diesel) as well as a co- generation power plant delivering 560 MW per annum to the national power grid.

Altona has since forged a Joint Venture with CNOOC New Energy Investment Co., Ltd. (CNOOC-NEI), a subsidiary of the China National Offshore Oil Corporation (CNOOC), to complete the Project Bankable Feasibility Study (BFS) and expedite the project’s development. CNOOC-NEI owns 51% of the

Arckaringa Unincorporated Evaluation Joint Venture (AUEJV) and Altona 49%. Under the terms of the AUEJV, the Australian subsidiary of CNOOC-NEI (CNOOC New Energy International (Australia) Pty Ltd or CNOOC- NEIA) will fund the BFS for the Arckaringa Project and will act as the operator, not

only to carry out the staged evaluation work under the BFS, but also to take responsibility for assessing the full potential of the coal resource and bring projects to development.

Permits and approvals are now in place for a drilling programme that will proceed when drilling contractors and other service providers have been selected. The key objectives of the programme are:

•  to extract bulk samples of coal to test

and extend the coal quality profile

•  to complete in-fill coal and coal geotechnical boreholes to facilitate open cut mine design (especially in an area covering the potential initial box cut)

•  to complete hydro-geological test wells to enable the refinement of the existing hydro-geological model and underpin a groundwater management plan.

Attaining these objectives is essential to the progress of the Bankable Feasibility Study (BFS) for the Arckaringa CTL and Power Project.

Unconventional Hydrocarbon Resource

Targeted

Mined coal to liquids (Figure 4.29)

Coal Deposits

Westfield - EL 4511 (England Hill Area) Wintinna - EL 4512 (Arckaringa Area) Murloocoppie - EL 4513 (Copper Hill Area)

Target Formation

Upper Mount Toondina Formation (Wintinna

Deposit)

26°0'S

133°30'E

NORTHERN TERRITORY

135°0'E

SC05/1

4

26°0'S

136°30'E

SC97/3

SC10/5

Tjayiwara

Unmuru

Irrwanyere  Mt Dare

Witjira (NP)

SC96/3

Eringa Part A

The Wangkangurru/ Yarluyandi

SP11/1

SC94/2

SC96/3

Proceeding  Simpson Desert (RR)

Wangkangurru

De Rose Hill  De Rose Hill

Eringa

SC96/3

Eringa No. 2

and

/Yarluyandi Petroleum Conjunctive ILUA (SI2011/023)

27°0'S

Anangu  Pitjantjatjara

Yankunytjatjara Lands

Mintabie "	"

27°0'S

Marla

SC97/9

Yankunytjatjara Antakirinja

EL 4513

Wangkangurru/Yarluyandi

" Oodnadatta

Oodnadatta

28°0'S

28°0'S

Far North

PRESWA

EL 4511 	EL 4512

"

29°0'S

Emu

Maralinga

Tjarutja  Lands

Arckaringa  Basin

Tallaringa (CP)

Coober Pedy

" Coober  Pedy

Lake Eyre (NP)

SC98/2

29°0'S

Arabana  People

Wabma  Kadarbu  Mound  Springs (CP)

Anangu  Pitjantjatjara

Yankunytjatjara Lands

Woomera Prohibited Area

30°0'S

30°0'S

ga

SC95/7

Olympic Dam

Andamooka

Antakirinja Matu-Yankunytjatjara

SC96/4	"

Barngarla 	"

31°0'S

Yellabinna (RR)

SC12/2

Naley

Yalata

Yellabinna (WA)

"

Tarcoola

Roxby Downs

31°0'S

" Glendambo

"

Woomera

Aboriginal Reserve

Boondina (CP)

SC06/1

Yumbarra (CP) Far West Coast

Yumbarra (CP)

Koonibba  Aboriginal Community

Lake Gairdner (NP)

SC97/7

Gawler  Ranges

SC09/1

Kokatha

Uwankara

132°0'E

Chadinga (CP)

133°30'E

Pureba (CP)

135°0'E

136°30'E

0 	50 	100 	150 km

REG 204218-014

Mineral exploration tenement

Mineral Exploration Licence (EL)

Restricted areas

MGA Zone 53

Indigenous Land Use

Agreement (ILUA)

Determinations of Native Title

Coalfield

Highway

Iron Knob

Port Augusta

Aboriginal Lands

Commonwealth land

Prescribed Wells Areas (PRESWA) Woomera Prohibited Area

National Estate area

Precious stones field

Parks with petroleum exploration access

Parks with no petroleum exploration access

Native Title Claims

SC96/4,

Barngarla Native Title Claim

SC97/3,

The Wangkangurru/Yarluyandi

Native Title Claim

SC06/1,

Far West Coast

SC09/1,

Kokatha Uwankara

Native Title Claim

SC10/5,

Tjayiwara Unmuru

Native Title Claim

SC12/2,

Naley Native Title Claim

SP11/1,

!  !  !  !  !  !  !  !

!  !  !  !  !  !  !  !

Roads

Railway

!  !  !  !  !  !  !  !

De Rose Hill

!  !  !  !  !  !  !  !

Figure 4.26	ELs 4511, 4512 and 4513, land access and Native Title.

QUATERNARY

Alluvium

Seam Name

CRETACEOUS

Bulldog Shale

Figure 4.28	Wintinna deposit cross - section.

204210-105

Cadna-Owie Formation

Coal Type

Sub-bituminous

Seam Description

JURASSIC

Algebuckina

Sandstone

Wintinna Open Cut Mine Target Area: 8 main seams (Figures 4.27 and 4.28) with combined 20-30 m thickness within 40-50 m vertical interval. Seams are un-weathered and generally flat lying with no structural complexities.

Unconformity

Isolation

EARLY PERMIAN

Upper Mt Tooddina Formation

Perentie Upper Perentie Goorikiana Upper

Goorikiana

Mirackina Upper

Mirackina

Arckaringa

Mt Barry Nilkinina Waddikee

0 		20 metres

Figure 4.27	Typical Stratigraphy, Wintinna coal deposit.

Overburden Thickness

145-210 m

Coal Quality Data

Wintinna Deposit Typical Raw Coal Quality

(average all seams)

Total Moisture (as received)                    36.6% Inherent Moisture (air dried basis)           13.2% Ash (air dried basis)                                   10.5% Volatile Matter (air dried basis)               32.5% Total Sulphur (air dried basis)                   1.64% Chlorine (dry, ash free basis)                   0.05% Specific Energy (air dried basis)      23.6 Mj/kg Relative Density                               1.25 (g/cc)

Sources: Statement of Coal Resources Wintinna July 2008, McElroy Bryan Geological Services (MBGS) Wintinna Resource Coal Quality Review, July 2008, A and B Mylec (for MBGS)

4

Combined

Cycle

Power Plant

Syngas

LP Steam

CO+2H

Tail Gas

2

Coal

Diesel

0

Gasification

Clean

2 	Up

Sulphur

Fischer-Tropsch

Synthesis

Refining

Naphtha

Other fuels

2

By Product

Insert Slag	CO

Figure 4.29	CTL conversion process.

Coal Resource

| Deposit | Million Tonnes (non JORC) |
| --- | --- |
| Measured1 Indicated1 Inferred1 Total |  |
| Wintinna 1,150 750 2,000 3,900 |  |
| Westfield 100 200 500 800 |  |
| Murloocoppie 250 300 2,600 3,150 |  |
| 7,850 |  |
| Million Tonnes (JORC compliant) |  |
| Million Tonnes (JORC compliant) 187 650 450 1,287 |  |

1 Based on SA Dept. for Minerals and Energy standards in the 1980s.

The Wintinna Deposit coal resource statement prepared by McElroy Bryan Geological Services in 2008 incorporates the results of Altona’s 2007/08 drilling programme. The report shows a JORC compliant resource estimate of 1,287 Mt, of which 837 Mt is in the Measured and

Indicated categories. The resource estimate covers only about 25% of the known area of the Wintinna deposit.

One tonne of Wintinna coal yields one barrel of liquids plus power and industrial products. The JORC compliant coal resource is therefore equivalent to approximately 1.2 billion barrels of liquid hydrocarbons.

Conversion Process

The Coal-to-Liquids (CTL) process consists of

three main steps (Figure 4.29) in which run of mine solid coal feedstock is converted

to higher value product using Gasification and Fischer-Tropsch (FT) processes. These are:

•  Synthesis Gas Generation

•  Liquid Synthesis using FT process

•  Products Upgrade

Large parts of the gasification and FT processes are exothermic. The Arckaringa Project will capture this heat to generate electricity and export it to the national grid. The nearest point of the SA electricity grid is some 350 km to the south at Roxby Downs/ Olympic Dam.

The Arckaringa Project is best summarized as a conventional open cut coal mine followed by processing and an oil refinery/petro- chemicals plant with efficient integrated combined cycle electricity generation which captures and uses waste heat.

Input

Coal feedstock: 10 Million Tonnes pa

Products

Fischer Tropsch Liquids:

•  10 Million Barrels pa (approx 80 % FT

diesel, 20% Naptha)

Power (Steam and Gas Turbines in Combined

Cycle Gas Turbine configuration):

•  1,140 MW (Gross)

•  562 MW (Net Export)

Environment And Land Access

Mining Act 1971 - Exploration

Amendments to the Mining Act that came into force on 1 July 2011 require the holder of an exploration licence (EL) to have

an approved program for environment protection and rehabilitation (PEPR) in place prior to the commencement of any exploration activities on an EL (Mining Act

1971 Part 10A, and Mining Regulation 114) (http://minerals.pir.sa.gov.au/licensing_and_ regulation/mining_operations).

A PEPR should identify all relevant environmental, social and economic impact events that may result from the proposed exploration activities and how each of

the identified impacts will be managed or

avoided.

DMITRE has developed a generic program

for environment protection and rehabilitation for low impact mineral exploration in South Australia which must be adhered to unless

an alternative low impact exploration PEPR has been developed and approved by the Minister. For all other activities (not within

the scope of the generic PEPR) a PEPR in accordance with Part 10A of the Act is required. Under Regulation 114 (Transitionary Provisions) an exploration work approval (EWA) for drilling/use of declared equipment, or a declaration of environmental factors (DEF) for exploration in sensitive areas (in accordance with EL conditions) are deemed to be a PEPR until 1 July 2014 unless otherwise required by the Minister.

It is intended that over the next 3 years a PEPR as required under the Act will eventually replace the current EWA and DEF formats. Guidelines and determinations for the development of this PEPR are currently being developed by DMITRE.

Exploration Compliance Reports (ECRs) are required as a condition of PEPR approval.

The Water Affecting Activity (WAA) permit

- required to cover the hydro-geological test and other drill holes currently planned, pursuant to the terms of the PEPR - was recently granted by the South Australian Arid Lands Natural Resources Management Board under the newly expanded and strengthened regulations governing water affecting activities in the Arid Lands regions of South Australia.

Following discussions with the Native Title claimant group Antakarinji Matu- Yankunytjatjara Aboriginal Corporation (AMYAC) and its representatives, the Work Area Clearance (WAC) process has also been formally completed.  The discussions have been ongoing since October 2011

and led to a formal acceptance by AMYAC on 12 July 2012 that the drilling programme design and drill hole locations meet the terms of the WAC (as required under the PEPR granted by DMITRE). As a result, all drill hole locations are now cleared for drilling.

The WAC process involved the relocation and/or redesign of six of the proposed maximum 31 drill holes to protect aboriginal heritage concerns during, and as a result

of, the drill programme . During discussions, AMYAC and the Arckaringa JV partners have agreed to work together to achieve and implement a framework or Memorandum

of Understanding (MOU) that can lead to a Native Title Agreement to cover respective rights, responsibilities and economic opportunities during any future mining and development project at Wintinna.

Mining Act 1971 - Mining

There is a two-stage authorisation process for mining in South Australia. (http://minerals.pir. sa.gov.au/licensing_and_regulation/mining_ operations)

Applications for a Mining Lease (Mineral (ML) or Extractive Mineral Lease (EML)), Retention Lease (RL) or Miscellaneous Purposes Licence (MPL) must be supported by a ‘Mining Lease Proposal’.

Once a tenement is granted, a Program for Environment Protection and Rehabilitation (PEPR) is required to be approved before mining may commence.

Appropriate documentation is required to be prepared and submitted to DMITRE for approval. The purpose of the documentation is to provide a comprehensive and detailed description of environmental, social and economic risks and benefits of the proposed operation so that stakeholders and DMITRE can make an informed, risk-based and balanced judgment about the proposed operation.

The Mining Regulation and Rehabilitation

Branch of DMITRE regulates all operating

mine and quarry sites. Each mining operation has a DMITRE Mining Compliance officer who is responsible for ensuring the site is operated in compliance with Lease conditions and

it’s Program for Environment Protection and Rehabilitation (PEPR). The provision

for a PEPR under the amended Mining Act commenced on 1 July 2011. This replaced the regulation under the Mining Act relating to the requirement for a Mining and Rehabilitation Program (MARP).

Publically available documents relating to all mines in South Australia are available from the South Australian Resources Information Geoserver (SARIG). Go to the top left widget, select Databases, then Publications and Reports from the drop down list.

Environmental Protection, Biodiversity and

Conservation Act 1999

Potential impacts on water resources from coal mining are a focus of the Commonwealth Government’s Independent Expert Scientific Committee and the

National Partnership for Coal Seam Gas and Large Coal Mines agreed between

the South Australian State Government and

the Commonwealth Government in 2012.	4

Pursuant to this partnership plans are already in place for:

•  regional studies of aquifers of the Arckaringa Basin and overlying younger basins; and

•  the South Australian State Government

to be accredited as a one-stop-shop

for the State and Federal governments’

environmental assessments pursuant to the Commonwealth Government’s Environmental Protection, Biodiversity and Conservation Act 1999; and

•  referring the environmental assessment of coal mining in the Arckaringa Basin to the Commonwealth Government’s Independent Expert Scientific Committee, to underpin informed regulation.

Forward Plan (Indicative Only)

2012 - Bankable Feasibility Study

2013 - Bankable Feasibility Study

2014 - Approvals and Financing

2015 – 2018 - Construction

Contact Details

Name:	Chris Schrape - Managing Director, Altona Energy Plc (Director, Arckaringa Energy Pty. Ltd)

Phone:	+61 8 8110 2500

Website:  www.altonaenergy.com.au

4.2.2 Walloway UCG Project

Licence and Licence Holder

PEL 120

SAPEX Ltd* (100%)

Wholly owned subsidiary of Linc Energy

*Denotes operator

Location

Walloway Basin, South Australia (Figure 4.30).

Project Summary

Linc Energy, as operator of PEL 120 in the

Walloway Basin located approximately

90 km south east of Port Augusta, completed a 15 well drilling program near Orroroo

(Figure 4.31) between August 2009 and March 2010 to assess the underground Coal Gasification (UCG) of Tertiary lignites (Figure 4.32).

The first 5 wells intersected coal at depths of between 200 and 293 m with coal seam thicknesses of up to 23 m. On completion of these wells Linc Energy commissioned Xenith Consulting Pty Ltd who determined

that, in accordance with the JORC Code, a

significant coal mineralisation target of 1.0 to

1.3 billion metric tonnes is indicated.

Unconventional Hydrocarbon Resource

Targeted

•  Underground Coal Gasification

(Figure 4.33)

Target Formations

•  Walloway Rider coal seam (UCG)

•   Walloway coal seam (UCG)

Target Depths

•  Walloway Rider coal seam: ~240 m

•  Walloway coal seam: 200-293 m

Target Thickness

•  Walloway Rider coal seam: ~3 m

•  Walloway coal seam: 8-23 m

Exploration Results To End 2011

•  Completed 15 wells in the Orroroo area

(Orroroo 1 to 15 wells)

Environment and Land Access

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound, safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats

and risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be approved for compliance before a regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following

link: http://www.pir.sa.gov.au/petroleum/ environment/register/seo,_eir_and_esa_ reports

Forward Plan

2012 - 2013: To be supplied

Contact Details

Name:	Ryan Koch

Phone:	08 8405 8708

Website:  www.lincenergy.com

The Dutchmans

Stern (CP)

138°0'E

"

SEE Figure 4.30

SC11/1

139°0'E

SC99/1

Adnyamathanha No.1

140°0'E	4

Quorn Adnyamathanha #4

SC10/2

Davenport

Port Augusta "

Native Title Claim

Ngadjuri Nation

PL2

Winninowie (CP)

Mount  Brown  (CP)

Mount

"

Orroroo

Yalpara  (CP) Black Rock (CP)

"

Yunta

SC12/1

Wilyakali

Remarkable (NP)

"

Melrose

Walloway Basin

Whyalla

33°0'S

(CP)

"

Whyalla

"

Port

Baroota

Telowie Gorge (CP)

"

Peterborough

33°0'S

Pualco

Range (CP)

SC96/4

Barngarla

Native Title

Claim

Munyaroo (CP)

Germein

"

Port Pirie

SC96/5

Nukunu Native

Title Claim

" Jamestown

PL1

Pandappa  (CP)

Caroona Creek (CP)

Clements

Gap (CP)

Mokota (CP)

SC11/2

PEL

120

Clare

"

Burra

"

Ngadjuri  Nation #2

Red Banks (CP) Hopkins

Creek (CP)

" Wallaroo

34°0'S

" Kadina

" Lochiel

Spring Gully (CP)

White  Dam  (CP)

34°0'S

Morgan "

" Moonta

Moonta

Port

Clinton (CP)  " Wakefield

PL1

Kapunda

Roonka (CP)

Morgan (CP)

Wills Creek (CP)

" Maitland

"

Nuriootpa

SN94/1

Brookfield (CP)

PL6

)	Point Pearce

SC00/1

Kaurna Peoples

Gawler

" " Penrice

Kaiserstuhl

Swan

Reach

Bakara

(CP)

Native Title Claim

Northern Adelaide

Plains PRESWA

" 	Gulf

" 		(CP) Hale  (CP)

Para Wirra  (RP)

PL13

Central

PL6

Swan Reach (CP)

Ridley (CP)

Mallee

PRESWA

Minlaton

St Vincent

SC10/3

Ramindjeri

Adelaide

Port Gawler (CP)

"

Minlacowie (CP)

138°0'E

ADELAIDE "

PRESW139A°0'E

Mannum

Lowan  (CP)

REG 204218-039

0	20	40	60	80 km

MGA Zone 54

CSG exploration wells

Selected exploration tenement

Petroleum Exploration Licence (PEL)

Native Title Claims

SC96/4, Barngarla Native Title Claim SC96/5, Nukunu Native Title Claim SC99/1, Adnyamathanha No.1

SC00/1, Kaurna Peoples Native Title Claim

SC10/2, Ngadjuri Nation

SC10/3, Ramindjeri

SC11/1, Adnyamathanha #4 Native Title Claim

SC11/2, Ngadjuri Nation #2

SC12/1, Wilyakali

SN94/1, R J Howard Investments

Restricted areas

Aboriginal Lands

Commonwealth land

Prescribed Wells Areas (PRESWA) Parks with petroleum exploration access Parks with no petroleum exploration access

Pipeline Licence (PL)

Gas

Liquids

Highway

Roads

Railway

Figure 4.30	PEL 120, land access and Native Title.

32°20'S

138°20'E

138°30'E

138°40'E

138°50'E

139°0'E

SC11/1

Adnyamathanha  #

Native Title Claim

PEL

120

Walloway

32°20'S

Basin

32°30'S

Yalpara (CP)

SC99/1

32°30'S

Adnyamathanha  No.1

SC96/5

Nukunu Native

Title Claim

2

6	7   8

9

1	3  10

5

SC10/2

32°40'S

Ngadjuri  Nation

32°40'S

11	4  12

13	14

15

Black Rock (CP)

138°20'E

138°30'E

"

Orroroo

138°40'E

138°50'E

SC11/2

Ngadjuri  Nation #2

139°0'E

REG 204218-040

0 	20 km

MGA Zone 54

CSG exploration wells

Selected exploration tenement

Petroleum Exploration Licence (PEL)

Restricted areas

Parks with no petroleum exploration access

Native Title Claims

SC96/5, Nukunu Native Title Claim

SC99/1, Adnyamathanha No.1

SC10/2, Ngadjuri Nation

SC11/1, Adnyamathanha #4 Native Title Claim

SC11/2, Ngadjuri Nation #2

Figure 4.31	Location of drillholes, PEL 120 Walloway Basin.

| WELL ORROROO 2 |  |  | WALLOWAY BASIN |  |  |  |  | PEL 120 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EASTING | 279787 | NORTHING |  | 6390380 | ELEVATION | 362m | COMPILED BY S KWITKO |  |
| DATE SPUD | 03/09/09 | TD DRILLED |  | 289.3m GL | TD LOGGED | 285.4m GL | DATE SEPT 2009 |  |

4

Depth

0

1

GRA

API

LSDG

g/cc

200

0

3

DTP

μs/ft

RLLD

LITH

200

WELL

CONSTRUCTION

Description

m GL

CALI

0	75

mm 	200          1

OHMM

1000

CLAY - red brown, soft

CLAY (70%) - red brown, soft GRAVEL (30%) - sub-angular to angular, poorly sorted, with pebbly lithics

CLAY - red brown, soft

CLAY (50%) - red brown, soft GRAVEL (50%) - angular, poorly sorted, with pebbly lithics

50

CLAY - light yellow to khaki, stiff, puggy, mottled

100

CLAY - grey to red-brown, soft, very sticky

CLAY - grey, soft, very sticky

150

CLAY, SILT, SAND - light grey soft clay and silt interbedded with pale grey, very fine grained sand

CLAY - medium grey to brown, soft

CLAY, SAND - light grey, soft clay interbedded with pale grey, very fine grained, subrounded, well sorted sand

200

CLAY - grey, soft, becoming carbonaceous towards base

COAL - lignite, dark brown

CLAY - medium grey, soft, carbonaceous fragments

250

CLAY, SILTSTONE - medium grey clay interbedded with bluish grey siltstone

SILTSTONE - bluish grey hard siltstone interbedded with minor dark shale bands

300

Figure 4.32	Stratigraphic section, Orroroo 2, Walloway Basin.

Figure 4.33	UCG process

4.2.3 Kingston Project Licences And Licence Holders PEL 127, EL 4576

Hybrid Energy Australia Pty. Ltd. - fully owned

subsidiary of Strike Energy Limited* (100%)

*Denotes operator

Location

Otway Basin South Australia (Figure 4.34).

Project Summary

In 1979, Western Mining Corporation Holdings Limited (WMC) discovered lignite in the Tertiary Dilwyn Formation on the northern margin of the Otway Basin (Figures 4.35

and 4.36). An extensive drilling programme comprising 950 holes (92 cored) followed over the next seven years, and a 985 million tonne deposit was delineated. The Kingston lignite was considered a feedstock for a proposed State power generation plant and comprehensive geological, geotechnical, hydrological, environmental and mining feasibility studies were undertaken. Pilot

combustion tests were conducted on bulk samples of the lignite extracted by 1 m diameter coring.

Strike Energy acquired the Kingston

lignite asset in 2005, and commenced an investigation of the resource for combustion and gasification potential.

The WMC drilling was sufficient for a JORC compliant resource estimate of 578 Mt (measured and indicated), determined

in 2007. Preliminary mining scope and groundwater studies followed in 2008/2009, with combustion and gasification test work

on the coals completed in 2011. The intensive gasification test work undertaken by the University of Adelaide confirmed that the Kingston lignite can be successfully gasified using fluid bed gasification technology to deliver good quality syngas that is high in hydrogen (Figure 4.37). Bench scale testing was undertaken to determine the optimal bed material and temperature operating

window, and semi-industrial scale testing was undertaken to prove that a continuous operation can be maintained and also demonstrate the range of gas compositions produced.

The project work completed by Strike Energy has confirmed that the Kingston Lignite Deposit can be mined, combusted, and gasified to produce syngas with the potential to manufacture high value products such as liquid fuel, urea, ammonia and methanol. Strike is currently seeking a partner for the advancement of the project.

Unconventional Hydrocarbon Resource	4

Targeted

Mined coal to synthesis gas (Figure 4.38)

Coal Deposits

Kingston Lignite Deposit

Target Formation

Dilwyn Formation

Coal Type

Lignite

140°0'E

Mallee PRESWA

141°0'E

Mount Boothby (CP)

Ngarkat (CP)

36°0'S

Coorong (NP)

SC98/4

Ngarrindjeri and Others

Native Title Claim

Messent

(CP)

Martin

Washpool (CP)

Gum Lagoon (CP)

36°0'S

Tintinara-Coonalpyn

PRESWA

Kelvin Powrie (CP)

" Keith

Tatiara PRESWA

Coorong and Lakes

Alexandrina and Albert

PEL 127

Tilley

Swamp (CP)

Gum Lagoon (CP)

Mount

Monster (CP)

Poocher Swamp (GR)

Aberdour (CP)

Bordertown

"

EL 4576

PL13

Wolseley Common (CP)

Custon (CP)

Jip Jip (CP)

Desert Camp (CP)

VICTORIA

Pine Hill Soak (CP)

Hanson Scrub (CP)

Padthaway "

Padthaway (CP) Bangham (CP)

0	10	20	30 km

MGA Zone 54

Padthaway

PRESWA

Talapar (CP)

Geegeela (CP)

Kingston

Grass Tree (CP)

Rosetown, Grave Reserve

Kingston "

Butcher Gap (CP)

Mount Scott (CP) 	Fairview (CP)

Mullinger

Swamp (CP)

37°0'S

Vivigani Ardune (CP)   "

37°0'S

Naracoorte "

Naracoorte

Otway  Basin

Kungari (CP)

Lucindale

Caves (NP)

Bool and Hacks

Guichen Bay (CP)

Lower Limestone Coast PRESWA

Lagoons

Hacks Lagoon (CP)

Robe

"

Woakwine (CP)

140°0'E

Big Heath (CP) Mary Seymour (CP)

Bool Lagoon

(GR)

141°0'E

Selected exploration tenements

Petroleum Exploration Licence (PEL) Mineral Exploration Licence (EL)

Gas pipeline

Coalfield

Native Title Claims

SC98/4,

Ngarrindjeri and Others

Native Title Claim

Restricted areas

Aboriginal Lands

Prescribed Wells Areas (PRESWA) Ramsar site

Parks with petroleum exploration access

Parks with no petroleum exploration access

REG 204218-019

Highway

Roads

Railway

Figure 4.34	PEL 127 and EL 4576, land access and Native Title.

0 m	Seam Description

RECENT

Moulineaux

Sand

The lignite deposit occurs in a NE trending elongate depression 27 km long and up to

Padthaway

FM

5 km wide. The lignite lies predominantly in one horizon, occurring as a single seam in

10	the south of the deposit and with partings

QUATERNARY

PLEISTOCENE

Bridgewater

FM

in the north (Figure 4.36). Maximum seam thickness is 9 m. In the north of the deposit a thickness of 12 m is attained where the main seam coalesces with an overlying less well

20

Coomandook

Formation

developed lignite horizon.

Overburden Thickness

20-80 m

OLIGACENE

Gambier

Limestone

30	Coal Quality Data

Kingston Lignite Deposit typical raw coal quality:

•  Moisture (as received):	53.3%

Narrawaturk

Marl

40	•  Ash (dry basis):	14.9%

•  Volatile Matter (dry basis):	46.9%

•  Sulphur (dry basis):	2.9%

•  Sodium (dry basis):	0.85%

50

TERITIARY

Mepunga

•  Chlorine (dry basis):	0.15%

•  Specific Energy (dry basis):	22.7 MJ/kg

•  Relative Density:	1.2

60	Coal Resource

| Kingston Deposit Strip Ratio | Million Tonnes (JORC compliant) Measured Indicated Total |
| --- | --- |
| < 5:1 217.1 4.5 221.5 |  |
| 5 to 10:1 229.9 20.8 250.7 |  |
| >10:1 76.5 29.5 106.0 |  |
| Total 523.5 54.8 578.3 |  |

EOCENE

Dilwyn Formatin (Fm)

70	MDOA Seam

ORDOVICIAN CAMBRIAN

Kanmantoo Group

80

NB. Thickness of formations diagrammatic only

Figure 4.35	Lignite seams in the Tertiary Dilwyn

Formation, Kingston deposit.

MDOB Seam

Input

Lignite from conventional open cut mining

Processing

Fluid bed gasification technology.

Product

Hydrogen rich synthesis gas, suitable for the manufacture of a range of products such as urea, ammonia, methanol or liquid fuel.

Environment And Land Access

Petroleum and Geothermal Act 2000

Figure 4.36	Cross section showing the lignite lobes of the Kingston deposit.

Figure 4.37    Effect of steam/fuel ratio on gas yield (L/kg) determined from semi-industrial scale gasifier (air/fuel = 2.3, bed T = 850ºC, 5% clay addition).

Grant of a petroleum licence does not enable (approve) regulated activities within those tenements. Approval for an activity (e.g. geophysical surveys or

drilling) is a separate process that is subject to a separate, rigorous assessment and consultation process, so that operations

are planned and regulated to be sound,

safe and meet community expectations for outcomes. An Environmental Impact Report (EIR) must be prepared, in consultation with stakeholders and relevant government agencies, addressing potential threats

and risks to the environment, and the extent to which these threats are likely and manageable. A Statement of Environmental Objectives (SEO), developed through an open, consultative process and based

on information in the EIR, must then be approved for compliance before a regulated activity can proceed. Please see Chapter 5 for further details of the approval process.

Copies of EIRs and SEOs prepared by licensees for regulated activities, and Environmental Significance Assessments prepared by the Energy Resources Division can be downloaded via the following link: www.pir.sa.gov.au/petroleum/environment/ register/seo,_eir_and_esa_reports

Mining Act 1971 - Exploration

Amendments to the Mining Act that came into force on 1 July 2011 require the holder of an exploration licence (EL) to have

an approved program for environment

| Combined Cycle Power Generation |  |  |
| --- | --- | --- |

2

CO

n

| Carbon Dioxide Capture a Compressi |  |  |
| --- | --- | --- |
|  | d on |  |

| Lignite Preparati and Size Reductio | n |  |
| --- | --- | --- |
|  | n |  |

| Syngas Cleaning a Conditioni |  | nd |  |
| --- | --- | --- | --- |
|  |  | ng |  |

| Gasifier |  |  |
| --- | --- | --- |

| Lignite Mi and Wate Manageme | e r |  |
| --- | --- | --- |
|  | nt |  |

n	o

Sulfer

Recovery

2

CO to

Geosequestration

Processing

Electricity

| Product Recovery and Refining |  |
| --- | --- |

Naphtha

Diesel

Sulfer

Mining	Preparation	Gasification	Cleaning	Synthesis	Conversion

Final

Products

Figure 4.38 	Coal Gasification – Process and Products.

protection and rehabilitation (PEPR) in place prior to the commencement of any exploration activities on an EL (Mining Act

1971 Part 10A, and Mining Regulation 114) (http://minerals.pir.sa.gov.au/licensing_and_ regulation/exploration#pepr).

A PEPR should identify all relevant environmental, social and economic impact events that may result from the proposed exploration activities and how each of

the identified impacts will be managed or

avoided.

DMITRE has developed a generic program

for environment protection and rehabilitation for low impact mineral exploration in South Australia which must be adhered to unless

an alternative low impact exploration PEPR has been developed and approved by the Minister. For all other activities (not within

the scope of the generic PEPR) a PEPR in accordance with Part 10A of the Act is required. Under Regulation 114 (Transitionary Provisions) an exploration work approval (EWA) for drilling/use of declared equipment, or a declaration of environmental factors (DEF) for exploration in sensitive areas (in accordance with EL conditions) are deemed to be a PEPR until 1 July 2014 unless otherwise required by the Minister.

It is intended that over the next 3 years a PEPR as required under the Act will eventually replace the current EWA and DEF formats. Guidelines and determinations for the development of this PEPR are currently being developed by DMITRE.

Exploration Compliance Reports (ECRs) are required as a condition of PEPR approval.

Mining Act 1971 - Mining

There is a two-stage authorisation process for mining in South Australia. (http://minerals.pir. sa.gov.au/licensing_and_regulation/mining_ operations)

Applications for a Mining Lease (Mineral (ML) or Extractive Mineral Lease (EML)), Retention Lease (RL) or Miscellaneous Purposes Licence (MPL) must be supported by a ‘Mining Lease Proposal’.

Once a tenement is granted, a Program for Environment Protection and Rehabilitation (PEPR) is required to be approved before mining may commence.

Appropriate documentation is required to be prepared and submitted to DMITRE for approval. The purpose of the documentation is to provide a comprehensive and detailed

economic risks and benefits of the proposed operation so that stakeholders and DMITRE can make an informed, risk-based and balanced judgment about the proposed operation.

The Mining Regulation and Rehabilitation

Branch of DMITRE regulates all operating

mine and quarry sites. Each mining operation has a DMITRE Mining Compliance officer who is responsible for ensuring the site is operated in compliance with Lease conditions and

it’s Program for Environment Protection and Rehabilitation (PEPR). The provision

for a PEPR under the amended Mining Act commenced on 1 July 2011. This replaced the regulation under the Mining Act relating to the requirement for a Mining and Rehabilitation Program (MARP).

Publically available documents relating to all mines in South Australia are available from the South Australian Resources Information Geoserver (SARIG). Go to the top left widget, select Databases, then Publications and Reports from the drop down list.

The original Western Mining Corporation Kingston lignite project environmental assessment reports include:

•  Assessments Branch, 1984. Kingston Lignite Project. Assessment report, South Australian Department of Environment and Planning. South Australia. ISBN

0724387447

•  Kinhill Stearns, 1983a, Kingston Lignite Project: draft environmental impact statement. Prepared for Western Mining Corporation Limited by Kinhill Stearns. ISBN 0959267816

•  Kinhill Stearns, 1983b. Kingston Lignite Project: supplement to the draft environmental impact statement. Prepared for Western Mining Corporation Limited by Kinhill Stearns. ISBN 0949397032.

Twenty six public and eight government submissions were received in response to the Draft EIS (1983a). Issues raised in each

proponent or included in full as appendices in the Supplement (1983b). Hard copies of the reports and submissions held in various libraries across the country can be located through the National Library of Australia – go to http://trove.nla.gov.au/

Contact Details

Name:	Mr Andrew Hooper-Nguyen – Chief Financial Officer and Chief Commercial Officer

Phone:	+61 2 8261 1000

Website:  www.strikeenergy.com.au